1994-97 Honda Accord LX 4 door pin out

| B1 | Black/White | Ceiling Light |
| --- | --- | --- |
| B2 | Green/White | Passager Rear Door indicator |
| B3 | Green/Red | Passager Door indicator |
| B4 | Orange | Trunk Indicator |
| B5 | Green/Orange | Driver Door Indicator |
| B6 | Yellow | (+) No.1 (10A) fuse (Low Oil) |
| B7 | Red | Dash Light Dimmer |
| B8 | Red/Black | Combination Light Switch |
| B9 | Yellow/Green | Engine Coolant Temp. |
| B10 | Black | Ground |
| B11 | (BLANK) | (BLANK) |
| B12 | Blue | Tachometer |
| B13 | White/Green | Brake Light Indicator |
| B14 | Green/Yellow | Driver Rear Door Indicator |
| B15 | (BLANK) | (BLANK) |
| B16 | (BLANK) | (BLANK) |
| B17 | (BLANK) | (BLANK) |
| B18 | Yellow | (+) No.35 (15A) fuse (Hazard) |
| B19 | Yellow/Red | Low oil |
| B20 | White/Blue | Alternator |
| B21 | Blue/White | Hazard warning switch |
| B22 | Black/Yellow | (+) No.4 (7.5A) fuse (Charging) |

| A1 | Blue/Red | (+) No.9 (7.5A) fuse (Brake Check) |
| --- | --- | --- |
| A2 | Orange/White | Dimmer Relay (High Beam) |
| A3 | Red/Yellow | (+) No.19 (20A) fuse (High Beam Indicator) |
| A4 | Light Green/Red | MIL |
| A5 | Yellow | (+) No.1 (10A) fuse (MIL) |
| A6 | (BLANK) | (BLANK) |
| A7 | Green/Blue | Turn Signal Left |
| A8 | Green/Red | Brake Check + Brake System |
| A9 | Black | Ground (Turn Signal) |
| A10 | Red/Blue | Seat Belt |
| A11 | Green/Yellow | Turn Signal Right |
| A12 | Blue | SRS |
| A13 | (BLANK) | (BLANK) |
| A14 | (BLANK) | (BLANK) |

| C1 | Yellow/Blue | Fuel |
| --- | --- | --- |
| C2 | Light Green/Red | Low Fuel |
| C3 | Blue/Red | “D4" |
| C4 | (BLANK) | (BLANK) |
| C5 | Green/White | “P" |
| C6 | Green/Red | “R" |
| C7 | Green | “N" |
| C8 | (BLANK) | (BLANK) |
| C9 | Blue/White | ABS |
| C10 | (BLANK) | (BLANK) |
| C11 | Blue/Black | Cruise |
| C12 | Orange | Speedometer |
| C13 | Light Green/White | “1” |
| C14 | Green/Yellow | “2” |
| C15 | Light Green | ECM/TCM |
| C16 | Green/Blue | “D3" |

1998-99 Accord LX 4 Door

| B1 | Blue | Tachometer |
| --- | --- | --- |
| B2 | Blue/White | Speedometer |
| B3 | (BLANK) | (BLANK) |
| B4 | Red/White | Dash Light Dimmer |
| B5 | White/Red | Dash Light Dimmer |
| B6 | Green/Orange | MIL |
| B7 | Yellow/Red | Low Oil |
| B8 | White/Blue | Alternator |
| B9 | Black/Yellow | (+) No.6 (15A) fuse (Charging) |
| B10 | Red | Cluster Lights |
| B11 | Yellow | (+) No.9 (7.5A) fuse (Brake System) |
| B12 | (BLANK) | (BLANK) |
| B13 | (BLANK) | (BLANK) |
| B14 | (BLANK) | (BLANK) |
| B15 | (BLANK) | (BLANK) |
| B16 | Black | Ground |
| B17 | (BLANK) | (BLANK) |
| B18 | (BLANK) | (BLANK) |
| B19 | Pink | ECM/PCM (immobilizer) |
| B20 | White/Yellow | (+) No.13 (7.5A) fuse (Immobilizer) |
| B21 | White/Yellow | (+) No.13 (7.5A) fuse (Trip) |
| B22 | Red/Black | Combination Light Switch |

| A1 | Green/Yellow | Left Rear Door |
| --- | --- | --- |
| A2 | Green/Orange | Driver Door |
| A3 | (BLANK) | (BLANK) |
| A4 | (BLANK) | (BLANK) |
| A5 | (BLANK) | (BLANK) |
| A6 | Blue/Black | Cruise |
| A7 | Green | Passager Door |
| A8 | Green/White | Right Rear Door |
| A9 | Orange | Trunk |
| A10 | Black/White | Ceiling Light |
| A11 | White/Green | Brake Light |
| A12 | Orange/White | High Beam Indicator |
| A13 | Red/Yellow | (+) No.45 (20A) fuse (High Beam) |
| A14 | Green/Blue | Left Turn Signal |

| C1 | Light Green/Red | Low Fuel |
| --- | --- | --- |
| C2 | Yellow/Green | Coolant Temp |
| C3 | Red/Blue | Seat Belt |
| C4 | White | “R" |
| C5 | Brown | “1” |
| C6 | Pink | “D3" |
| C7 | Blue | “2” |
| C8 | Green/Yellow | Right Turn Signal |
| C9 | Blue | SRS |
| C10 | Blue/White | ABS |
| C11 | Yellow/Blue | Fuel |
| C12 | Green/Black | “D4" |
| C13 | (BLANK) | (BLANK) |
| C14 | Green/Red | Brake System Light |
| C15 | Black/Blue | “P" |
| C16 | Red/Black | “N" |

| Plug pin on 98-99 | Plug pin from 94-97 | Color |  |
| --- | --- | --- | --- |
| B1 | B12 | Blue | Tachometer |
| B2 | C12 | Orange | Speedometer |
| B3 | N/A | N/A | N/A |
| B4 | N/A | N/A | N/A |
| B5 | N/A | N/A | N/A |
| B6 | A4 | Light Green/ Red | MIL |
| B7 | B19 | Yellow/Red | Low Oil |
| B8 | B20 | White/Blue | Alternator |
| B9 | B22 | Black/Yellow | (+) No.4 (7.5A) fuse (Charging) |
| B10 | B7 | Red | Cluster Lights |
| B11 | B6 | Yellow | (+) No.1 (10A) fuse (Low Oil) |
| B12 | N/A | N/A | N/A |
| B13 | N/A | N/A | N/A |
| B14 | N/A | N/A | N/A |
| B15 | N/A | N/A | N/A |
| B16 | B10 | Black | Ground |
| B17 | N/A | N/A | N/A |
| B18 | N/A | N/A | N/A |
| B19 | N/A | N/A | N/A |
| B20 | N/A | N/A | N/A |
| B21 | A5 | Yellow | (+) No.1 (10A) fuse (MIL) |
| B22 | B8 | Red/Black | Combination Light Switch |

1994-97 to 1998-99 cluster swap

| Plug pin on 98-99 | Plug pin from 94-97 | Color |  |
| --- | --- | --- | --- |
| A1 | B14 | Green/Yellow | Left Rear Door |
| A2 | B5 | Green/Orange | Driver Door |
| A3 | N/A | N/A | N/A |
| A4 | N/A | N/A | N/A |
| A5 | N/A | N/A | N/A |
| A6 | C11 | Blue/Black | Cruise |
| A7 | B3 | Green/Red | Passager Door |
| A8 | B2 | Green/White | Right Rear Door |
| A9 | B4 | Orange | Trunk |
| A10 | B1 | Black/White | Ceiling Light |
| A11 | N/A | N/A | N/A |
| A12 | A2 | Orange/White | High Beam Indicator |
| A13 | A3 | Red/Yellow | (+) No.19 (20A) fuse (High Beam Indicator) |
| A14 | A7 | Green/Blue | Left Turn Signal |

| Plug pin on 98-99 | Plug pin from 94-97 | Color |  |
| --- | --- | --- | --- |
| C1 | C2 | Light Green/Red | Low Fuel |
| C2 | B9 | Yellow/Green | Coolant Temp |
| C3 | A10 | Red/Blue | Seat Belt |
| C4 | C6 | Green/Red | “R" |
| C5 | C13 | Light Green/White | “1” |
| C6 | C16 | Green/Blue | “D3" |
| C7 | C14 | Green/Yellow | “2” |
| C8 | A11 | Green/Yellow | Right Turn Signal |
| C9 | A12 | Blue | SRS |
| C10 | C9 | Blue/White | ABS |
| C11 | C1 | Yellow/Blue | Fuel |
| C12 | C3 | Blue/Red | “D4" |
| C13 | N/A | N/A | N/A |
| C14 | A8 | Green/Red | Brake Check + Brake System |
| C15 | C5 | Green/White | “P" |
| C16 | C7 | Green | “N" |